User Guide

s-SNOM Spectrum Calculator

1. Overview

The s-SNOM Spectrum Calculator is a web application for calculating near-field optical spectra using a spheroid model of scattering-type scanning near-field optical microscopy (s-SNOM).

The application allows users to calculate the complex near-field signal and display:

near-field amplitude |σₙ(ω)|
near-field phase arg(σₙ(ω))
dielectric permittivity of the material ε(ω)

The current version supports bulk samples, single-layer on substrate, and double-layer on substrate.

2. Basic workflow

A typical calculation consists of the following steps:

- Select the sample model.

- Enter tip parameters.

- Enter the reference material and harmonic number.

- Choose the frequency range.

- Select the calculation algorithm and discretization parameters (see 1 for more details).

- Select or upload the optical properties of the sample.

- Optionally upload an experimental spectrum for comparison with the calculated result.

- Press Calculate.

- View the calculated spectra.

- Save the results if needed.

3. Model selection

3.1. Bulk sample

Use this mode for a homogeneous semi-infinite sample. The sample can be isotropic or uniaxial with the optical axis orthogonal to the surface. In the case of a uniaxial sample, use ε∥ε⊥ as the dielectric permittivity of the sample.

3.2. Layered sample

Use this mode for thin isotropic layers on a substrate.

Two geometries may be available:

1 layer on substrate
2 layers on substrate

For one layer:

Tip
↓
Layer 1
↓
Substrate

For two layers:

Tip
↓
Layer 1
↓
Layer 2
↓
Substrate

The substrate is considered semi-infinite, so no substrate thickness is required.

4. Tip parameters

The Tip parameters section defines the geometry and optical properties of the spheroid approximating the s-SNOM tip.

Tip parameters are:

| Parameter | Symbol | Unit |
| --- | --- | --- |
| Curvature radius of apex | R | nm |
| Spheroid length | L | nm |
| Tapping amplitude | A | nm |
| Minimum tip-sample distance | H0,s | nm |
| Minimum tip-reference distance | H0,r | nm |
| Real part of tip permittivity | Re(εt) | — |
| Imaginary part of tip permittivity | Im(εt) | — |

All length parameters must be positive.

5. Far-field contribution

If the option Take into account far-field multiplier is disabled, the spheroid polarizability α is calculated. If this option is enabled, the calculated spectrum includes far-field reflection coefficient, σ~1+ςr2α, where ς is the indirect illumination coefficient.

When this option is enabled, the following parameters become active:

| Parameter | Symbol | Unit |
| --- | --- | --- |
| Indirect illumination coefficient | ς | — |
| Illumination angle | θ | degree |

6. Reference material

The near-field signal is normalized to a reference material.

The reference material is defined by a constant complex dielectric permittivity:

εref = Re(εref) + i Im(εref)

7. Demodulation harmonic

The field is demodulated at harmonic number n.

Typical values are:

n = 2
n = 3
n = 4

The calculated signal is denoted as:

σₙ

The output graphs show:

|σₙ|
arg(σₙ), rad

8. Frequency range

The Frequency range section defines the spectral range for the calculation.

The required parameters are:

| Parameter | Unit |
| --- | --- |
| Start frequency | cm⁻¹ |
| End frequency | cm⁻¹ |
| Number of points | — |

If a material is uploaded from a file, the selected frequency range must lie inside the frequency range provided in the file.

9. Sample material

The sample material can be specified in different ways.

9.1. Built-in material

Select a material from the built-in list.

The application automatically loads its dielectric permittivity:

ε(ω) = Re(ε) + i Im(ε)

The permittivity is shown on the graph after calculation.

9.2. Uploaded material file

The user can upload a text, CSV, or DAT file containing the material dielectric permittivity.

The file must contain at least three columns:

frequency_cm-1    Re_epsilon    Im_epsilon

Example:

1650    2.22    0.02
1651    2.23    0.02
1652    2.24    0.03

The frequency must be given in cm⁻¹.

The second and third columns must contain the real and imaginary parts of the dielectric permittivity.

The application interpolates the uploaded data onto the selected calculation frequency grid.

10. Layered sample material parameters

For layered samples, each part of the structure has its own material.

For one layer on substrate:

Layer 1 material
Substrate material

For two layers on substrate:

Layer 1 material
Layer 2 material
Substrate material

Each layer also has a thickness in nm.

Example:

Layer 1 thickness = 20 nm
Layer 1 material = PMMA
Substrate material = Si

For two layers:

Layer 1 thickness = 20 nm
Layer 1 material = PMMA

Layer 2 thickness = 50 nm
Layer 2 material = SiO2

Substrate material = Si

11. Calculation algorithm

The application may provide two calculation algorithms:

Fast
Accurate

Fast

The fast mode is suitable for quick calculations and for materials with positive dielectric permittivity, or for materials with a negative real part of the dielectric permittivity and a high imaginary part.

Accurate

The accurate mode is slower but more reliable for resonant materials with dielectric permittivity close to values around –1, where the near-field response changes rapidly. In this case, the discretization parameters should be defined manually. Increasing these parameters may improve numerical accuracy but also increases calculation time. The results converge to an exact solution of the electrostatic problem for sufficiently large N, M, and K.

Allowed limits:

N <= 500; M <= 100; K <= 50

12. Experimental spectrum

The user can upload an experimental spectrum and display it together with the calculated result.

Enable:

Show user's spectrum in the same plot

Then upload a file.

The supported format is:

frequency_cm-1    amplitude    phase

The experimental spectrum is displayed as uploaded and does not need to have exactly the same frequency grid as the calculated spectrum.

13. Running the calculation

After all required parameters are entered, press:

Calculate

After the calculation is complete, the application displays:

- dielectric permittivity;

- near-field amplitude;

- near-field phase.

If an error occurs, a message will explain what needs to be corrected.

Common errors include:

The selected frequency range is outside the uploaded permittivity data range.
Layer thickness must be positive.
N must be between 1 and 500.
M must be between 1 and 100.
K must be between 1 and 10.

14. Graphs

14.1. Dielectric permittivity

This graph shows:

Re(ε)
Im(ε)

For bulk samples, it shows the permittivity of the sample.

For layered samples, it may show the permittivity of:

Layer 1
Layer 2
Substrate

14.2. Near-field amplitude

This graph shows:

|σₙ|

If an experimental amplitude spectrum was uploaded, it is displayed on the same graph.

14.3. Near-field phase

This graph shows:

arg(σₙ)

If an experimental phase spectrum was uploaded, it is displayed on the same graph.

15. Comparing several calculated spectra

The application can keep several recently calculated spectra during the current browser session.

After a new calculation, the newest spectrum is shown by default.

Older spectra remain available in the graph legend but are hidden by default.

To show an older spectrum, click its name in the legend.

To hide it again, click its name again.

This allows quick comparison between calculations with different parameters.

The application may store only a limited number of spectra. When the limit is exceeded, the oldest spectrum is removed from the session history.

The session history is temporary and may be cleared when the page is refreshed or closed.

16. Saving results

After calculation, press:

Save results

The application downloads a CSV file containing the calculated data.

K. V. Voronin, I. H. León, R. Hillenbrand, and A. Y. Nikitin, “Quantitative Analytical Spheroid Model for Scattering-Type Scanning Near-Field Optical Spectroscopy.” Adv. Optical Mater.13, no. 31 (2025).